# EBN 111 – Summary

# Chapter 1

#### Charge and Current

- Charge is an electrical property of the atomic particles of which matter consists – measured in coulombs

  - Charge on 1 electron: -1,602 x 10-19C

  - 1C = 6.24 x 1018 electrons

  - Charge cannot be created/ destroyed, only transformed

- Current – rate of change of charge (Ampere)

  - i=qt

  - However, current is not always constant, therefore i=dqdt

    - Example 1 – Determine the current if qt=8t2+4t-2 at t=3

i=dqdt

i=ddt8t2+4t-2

i=16t+4

i=16×3+4

i=52A

    - Example 2 - Determine the current if qt=10sin⁡(120πt) at t=0.5

i=dqdt

i=ddt10sin⁡(120πt)

i=10cos120πt×120π

i= 1200πA

  - Similarly Q=t0ti dt

    - Example 1 – Find Q if it=20cos⁡(10t+π6) ) and between t(0) and t(1)

Q=t0ti dt

Q=t0t20cos⁡(10t+π6) dt

Q=-200sin⁡(10t+π6)01

Q=278,13C

- Direct current – current remains constant

- Alternating current – current that varies sinusoidally with time

#### Voltage

- Emf/potential difference/ voltage – the energy required to move charge through a conductor

- v=wq

- If v is not constant then v=dwdq

- Voltage can be positive or negative depending on how it is represented

  - As current moves through an element, the voltage drops or is used up

  - Thus the voltage “up current” will be higher (or lower if it is a cell) than that below the element

  - Thus if voltage is represented from above to below the element then it is positive and if it is represented from below to above then it is positive

- Voltage can also be represented positively or negatively according to whether it is supplied or used up buy the element

#### Power and Energy

- Power – the rate at which energy is expended (Watts)

  - p=dwdt=vi

  - Power calculated using p=vi – instantaneous power

  - Power is either positive or negative

    - Positive when

      - Energy is expended or used (as in a resistor)

      - Current enters the positive terminal of the element

    - Negative when

      - Energy is supplied (as in a cell)

      - Current enters the negative terminal of the element

  - Energy must be conserved, therefore total power of a system must equal zero

- Energy – the capacity to do work

  - Measured in joules or watthours

  - w=v dq

#### Circuit elements

- Active elements – elements that generate energy

+

-

+

-

Ideal independent source – supplies voltage or current of a specific quantity no matter what the rest of the circuit looks like

    - Symbol  -            voltage		      current

  - Ideal dependant source – supplies energy in quantities controlled by another element

    - Types

      - Voltage controlled voltage source (VCVS)

      - Voltage controlled current source (VCCS)

      - Current controlled voltage source (CCVS)

      - Current controlled current source (CCCS)

    - Symbol – same as independent with a diamond instead of a circle

- Passive – elements that is not capable of generating energy

# Chapter 2

#### Ohm’s Law

- Used to calculate resistance – the force that resists the flow of electric charge

- Can be calculated using:

  - R=ρlA

    - P – resistivity of the material

    - L – length of the resistor

    - A – cross-sectional area of the material

  - R=vi

  - R – measured in Ohms (Ω)

  - R is always positive

- Power in resistors

  - P=i2R

  - P=v2R

  - Power in a resistor is always positive

- Law only applies to linear resistors (where v α i)

- Open circuit – circuit with an element with resistance approaching infinity

- Short circuit – circuit with resistance approaching zero

- Conductance – the ability of an element to conduct electric current

  - Measured in mhos or Siemens (S)

  - Is the inversion of resistance  -  G=1R

#### Nodes, Branches and Loops

- Branch – a single element such as a voltage source or a resistor

- Node – a point of connection between two or more branches

  - Two elements are in series if they are connected by a single node

  - Two elements are in parallel if they are connected to the same two nodes

- Loop – any closed path in a circuit

  - Independent loops

    - Loop cannot be broken down into smaller loops

    - Contains an element not found on other independent loops

  - Calculate numbers using – b=l+n-1

    - Where l represents independent loops

#### Kirchhoff’s Laws

- Current law – the sum of all currents entering and leaving a node must be zero

  - Write down the current value for all known branches around the node

  - Make each of these positive or negative according to whether the current is entering or leaving the node through the branch

  - Add together equalling 0

Example:

-i1+i2+i3-i4=0

- Voltage Law – the sum of all voltages around a closed loop is zero

  - Start from any point and move clockwise

  - If current enters at the positive, add, if enters at negative, subtract

  - Add together, equalling zero

Example:

R1+R2-R3-C4=0

5×103i+10×103i-7,5×103i-45=0

7,5×103i=45

#### i=166,7A
Series resistors and voltage division

- Resistors in series carry the same current

  - ∴i=vReq

  - Req=R1+R2+R3

- Voltage division in series

  - vn=RnReq.v

#### Parallel resistors and current division

- Resistors have the same voltage as the whole parallel system

  - 1Req=1R1+1R2+1R3

- Current division

  - If there are only 2 resistors

    - i1=iR2R1+R2

  - If there are more than 2 resistors

    - G=1R

    - ∴Geq=G1+G2+G3

    - ∴in=iGnGeq

      - Convert all resistances into conductance

      - Find total G

      - Sub into formula

#### Wye-Delta Transformations

- Often is a more complex circuit, there are no obvious parallel and series resistors, however this can be changed to make it easier

- Circuits in a delta form can be converted to wye form and visa-versa

- Balanced resistors – all resistors have the same ohm value

  - Ra=Rb=Rc=R∆

  - R1=R2=R3=Ry

  - R∆=3Ry

Example:

R∆=3Ry

R∆=12Ω

∴Ry=4Ω

- Wye to delta

  - Ra=R1R2+R1R3+R3R2R1

  - Ra=all the products of possible 2 Rn valuesRn value opposite

Example:

Ra=R1R2+R1R3+R3R2R1

Ra=30.20+20.50+30.5030

Ra=103,33Ω

Rb=155Ω

Rc=62Ω

- Delta to wye

  - R1=RbRcRa+Rb+Rc

  - Ra=Product of adjacent resistorssum of all resistors in delta

Example:

R1=RbRcRa+Rb+Rc

R1=30.6030+60+10

R1=18Ω

R2=6Ω

R3=3Ω

# Chapter 3

#### Nodal Analysis

- Provides us with node voltages, using Kirchhoff’s current law

- Reduces the number of equations required to solve a circuit

- Steps

  - Select a node as the reference node (ground) – this node will have 0V as its potential difference

  - Assign voltages, V1,V2 etc. to each critical node (nodes with more than two branches)

  - Apply KCL to each of these named nodes – use ohms law to express the branch currents in terms of node voltages

  - Solve the simultaneous equations (this case used Cramer’s rule)

Example: Find the node voltages

V1:  	0=V1-V23-i+V1-V32

0=V1-V23-10+V1-V32

0=2V1-2V2-60+3V1-3V3

60=5V1-2V2-3V3

V3:  	0=V3-06+4i+V3-V12

0=V3-V12+4i+V36

0=6V3-6V1 +12V2+2V3

0=-3V1 +6V2+4V3

5-2-3-4-50-364V1V2V3=6000

∆=5.-5.4+-4.6.-3+-3.-2.0--3.-5.-3-   0.6.5-4.-2.-4

∆=-100+72+0+45-0-32

∆=-15

V2=∆2∆

V2=960-15

V2=-64V

V2:  	0=V2-V13-i+V2-04

0=V2-V13-4i+V24

0=-4V1+4V2-12.4.V24+3V2

0=-4V1+4V2-12.4.V24+3V2

0=-4V1-5V2

V1=∆1∆

∆1=60-2-30-50064

∆1=60.-5.4+0.6.-3+0.-2.0--3.-5.0-   0.6.60-4.-2.0

∆1=-1200

V1=-1200-15

V1=80V

V3=∆3∆

V3=-2340-15

V3=156V

#### Supernodes

- The enclosed voltage source connected between any two non-reference nodes and any elements connected in parallel with it

- This is useful as it reduces the number of nodes and simplifies the problem

- Also creates a new equation: v1-v2=2v0 (voltage across the supernode)

  - This is easier to work with then most other simultaneous equations

#### Mesh Analysis

- Mesh – loop that does not contain any other loops

- Mesh analysis – only applies to planar circuits

  - Planar circuit – can be drawn without any branches crossing

  - Nonplanar circuit – cannot be drawn without branches crossing

- Applies Kirchhoff’s  voltage law to find mesh currents

- Steps

  - Assign mesh currents i1, i2,…in to each of the meshes – draw them as a current flowing clockwise through each mesh

  - Apply KVL to each of the meshes using Ohms law to express the voltages in terms of currents

    - If the branch is only present in one mesh then the current value will just be i1, or whatever the mesh number is

    - If the branch is present in two meshes then the current is expressed as: i1-i2, where i1 is the mesh that you are currently working with and i2 is the other mesh

  - Solve the equations simultaneously

Example:

i0=i3

10i0=10i3

i1:0=-20+4i1-i3+2i1-i2

0=6i1-4i3-2i2-20

i1=46i3+26i2+206

i1=46i3+265828i3+2028+206

i1=1914i3+257

i1=1914(-5)+257

i1=-3,21A

i3:0=6i3+4i3-i1+8i3-i2

0=-4i1+18i3-8i2

0=-41914i3+257+18i3-85828i3+2028

0=-4i3-20

i3=-5A

i2:0=-10i3+2i2-i1+8i2-i3

0=-2i1-18i3+10i2

0=-246i3+26i2+206-18i3+10i2

0=-43i3-23i2-203-18i3+10i2

0=283i2-583i3-203

i2=5828i3+2028

i2=5828(-5)+2028

i2=-9,64A

i0=i3

i0=-5A

#### Supermeshes

- Supermesh – two meshes have a current source with or without other elements in series, in common

- Steps for super meshes

  - Assign the currents as normal for mesh analysis

  - Then combine the two meshes with the common source into one current

  - Then derive two equations from this supermesh

    - i1-i2=current flowing through the source

    - The general mesh analysis equation is terms of i1 and i2 for the large current

i1-i2=3A

0=2(i1-i3)+4(i2-i3)+8i2-6

6=2i1+12i2-6i3

0=2i3-i1+2i3+4i3-i2

0=-2i1-4i2+8i3

1-10212-6-2-48i1i2i3=360

i1=3,47A

i2=0,47A

i3=1,11A

#### Nodal Analysis by Inspection

- Only can be used if all sources are independent current sources

- Use the equation:

G1+G2-G2-G2G2+G3v1v2=i1-i2i2

  - Where the diagonal elements from the top left to the bottom right are the sum off all the conductances branching off the node

  - The other elements in the matrix are the value of the conductance directly between the two nodes

  - The current values represent the current entering (+) and exiting (-) the node – onlt applies to branches with current sources on them

- Equation can also look like:

G11G21G12G22v1v2=iin-ioutiin-iout

Example:

#### Mesh Analysis by inspection

- Only can be used with all sources as independent voltage sources

- Use the equation:

R1+R2-R2-R2R2+R3i1i2=v1-v2

  - Where the diagonal elements from the top left to the bottom right are the sum off all the resistances in the loop

  - The other elements in the matrix are the value of the resistances common between the two loops

  - Voltages represent the sum of all the voltage increases due to voltage sources within the loop

    - If current enters at – then sign is +

    - Enters at + then sign is –

Example:

#### Nodal vs Mesh Analysis

- If the question specifies then follow it, if not then look at the following factors

  - Weigh up whether there are fewer nodes or meshes and then chose the one with the least

  - If there are more parallel elements – use nodal

  - More series – use mesh

#### Transistors

- Work with a BJT (Bipolar Junction Transistor)  - most often made of layered materials (npn)

- Working with currents (mesh analysis)

  - Current Ic is increased by the current IB, like a valve

  - IC=βIB – where β is the common emitter gain (value of 50-100 usually)

  - IC=αIE – where α is the common base current gain (value: 0.98-0.99)

  - ∴β=α1-α

  - Only really use the β value in practise

- Working with voltages (node analysis)

  - First redraw the circuit so that it is in the equivalent network model

  - Use KVL to calculate the values

Example with Mesh analysis:

0=Ib10000+VBE+1+βIb200-5

0=Ib10000+0,7+101Ib200-5

Ib=0,14mA

0=-500βIb+12-2001+βIb-VCE

0=-50000.Ib+12-20200Ib-VCE

VCE=2V

Vo=200(1+β)Ib

Vo=2,87

Example with Nodal analysis:

IB=1V-0,7V120000=2,5μA

IC:   0=80IB+VC-1020000+VC20000

0=1600002,5×10-6+2VC-10

VC=3V=Vo

Io=Vo20000=0,15mA

# Chapter 4

#### Linearity

- A circuit in which the output is directly proportional to the input

  - The voltages and currents across the linear circuit will remain at a constant ratio

- Properties of linearity

  - Scaling – v=iR→kv=kiR

  - Additivity - v=i2+i1R=v2+v1

- Calculating values using linearity

  - Assume the final outcome (what you are calculating) to be 1 units or whatever is requested in value

  - Work backwards through the circuit using KVL and KCL for the assumed value until you reach a component with a fixed value

  - Find the ratio between the assumed outcome value for the fixed value component and its stated value

  - Use this ratio to calculate the correct value for the final outcome

Example:

- Assume I0 to be 1A

- Work backwards through the problem

  - Using current division – I0=I243+5+4=1

  - I2=3A

  - I2=I477+2+4||8=I4711.67

  - I4=5A=Is

- Create a ratio for assumed values

  - I0I4=15

- Use this ratio to calculate I0

  - 15×Is=I0

  - I0=3A

#### Superposition

- The voltage and current flowing through each element is made up of the sum of the influence of each independent source

- Is a calculation of the contribution of each independent source separately

- Steps for calculations

  - Turn off all independent sources except for one, then calculate the output asked in the question

  - Repeat step one for each independent source

  - Add each of the values calculated in step one and two together

- Turning off sources

  - Voltage sources are replaced with a wire and leave all elements in series as part of the circuit

  - Independent current sources are replaced with an open circuit meaning that all elements in series are excluded

Example:

0=-0.1vx+vx4+vx-2020

0=-2vx+5vx+vx-20

20=4vx

vx=5V

0=-4-0.1vx+vx20+vx4

4=-2vx+vx+5vx

4=4vx

vx=1V

vx=4+1

vx=5V

#### Source Transformations

- Involves the creation of an equivalent circuit with the same v and I values

- Voltage to Current

  - Change the resistor from series to parallel

  - Replace the voltage source with a current source with the current flowing I the direction that the positive terminal pointed

  - Use the equation is=vR

- Current to voltage

  - Change the resistor from parallel to series

  - Replace the current source with a voltage source

  - Use ohms law to calculate the new voltage value

#### Thevenin’s theorem

- A two terminal circuit can be replaced with an equivalent circuit made up of a voltage source in series with a resistor

  - Vth is the open circuit voltage at the terminals

  - Rth is the equivalent resistance at the terminals when the independent sources are turned off

- Method 1 – circuit contains no dependant sources

  - Remove the load part of the circuit and draw in the open circuit Vth

  - Simplify/ use KCL and KVL to determine the value of Vth

  - Turn off all the independent sources and calculate the equivalent resistance - Rth

Example:

Step 1:

Step 2: Find Vth

0=-3+V1-186+V110

0=-90+5V1-90+3V1

V1=22.5V

Vab=V1.410

Vab=9V

∴Vth=9V

Step 3:

6+6||4=3Ω

Final Circuit:

- Method 2 – With dependant sources

- Use the same method for determining Vth

- Dependant sources can’t be turned off thus they need to be kept

- Thus a Test source needs to be applied to the circuit either a

  - Voltage source if using node voltages is easier

  - Current source if mesh analysis would be easier

- Use the equation Rth=v0i0

  - Where V0 is either the 1V test source or the voltage across the 1A test current source

  - I0 is either the 1A test source or the current flowing through the 1V test source

Example:

Vx:

0=Vx5-1.5Ix+Vx-13

0=Vx5-1.5Vx-13+Vx-13

0=3Vx-7.5Vx-7.5+5Vx-5

Vx=-5V

V1:

0=-i0+1--53+14

i0=2.25A

Rth:

Rth=v0i0

Rth=12.25

Rth=0.4Ω

#### Norton’s Theorem

- Exactly the same as Thevenin’s theorem except resulting in a current source in parallel with a resistor

- Just the source transformation of Thevenin’s equivalent

- Use same steps as thevenin just short circuit a-b to find IN

Example:

Step 1: Determine IN

Thus Vx = 0 and the current flowing through 6Ω is 0A.

Isc = 10A

Using Method 3: short circuit terminal a-b and do not zero any sources. RN=Voc/isc

Voc: Use source transformation

0=6I-60+22I+2I

I=5V

∴Vx=Voc=10V

Isc:

Previously calculated to be 10A

RN=VocIsc

RN=10V10A

RN=1Ω

#### Maximum Power Transfer

- Applies when the whole circuit except for the load is replaced by the Thevenin equivalent

- Then the max power transferred to the load is:

  - P=i2RL=VthRth+RLRL

  - Simplified to: P=Vth24Rth=VL2Rth

  - The load resistor must equal thevenins resistor for maximum power transfer to apply

- Method – use thevinins rule to simplify the circuit then just apply the equations

# Operational Amplifiers

- Also called an op amp

- Behaves like a voltage controlled voltage source

- It is an active circuit element designed to perform mathematical operations such as addition, multiplication, differentiation etc.

- Made up of many different transistors and resistors inside the amplifier but not shown in the course

  - Connected up but the following pins

#### Non Ideal Op Amp

- Represented in the following way

Equivalent Circuit is:

Equivalent Circuit is:

- Parameters are given as:

- Thus the output current is related to the voltage across Ri

  - But it is limited to |Vcc| the maximum voltage that can be supplied by the op-amp

- Method of calculating

  - Replace the op-amp symbol with the equivalent circuit (make sure that the positive and negative terminals are connected in the right places)

  - Simplify the circuit graphically and by using source transformations, inserting all the given information

  - Solve as one would solve any other circuit problems

Example:  Find V0/Vs when A = 2 x 105, Ri = 2 MΩ, R0 = 50Ω.

Step 1:

Step 2:

0=V15 000+V1-Vs2 000 000+V1-V040 000

0=400V1+V1-Vs+50V1-50V0

Vs=451V1-50V0

0=V0-(2×105)(Vs-V1)50+V020 000+V0-V140000

0=800(V0-200 000Vs+200 000V1)+2V0+V0-V1

160 000 000Vs=803V0+159 999 999V1

451Vs=0.00226V0+451V1

∴450Vs=50.00226V0

V0Vs=8.9996

#### Ideal Op Amps

- Has the ideal characteristics as seen in the table above

- Since Ri = ∞

  - i1=i2=0

  - vd=v1-v2=0

  - v1=v2

Example: Assume Vs = 1V and find V0

0=15000+1-v140000

v1=9V

∴V0=9V

#### Inverting Amplifier

- This form of an op amp supplies similar voltage to the circuit just polarised

- Must have the following characteristics

  - An equivalent resistance in series with Vi

  - An equivalent resistance: Rf

  - An inverting input or output of 0V

- Thus the following equation applies:

v0=-RfR1vi

- Special case where

  - Current is supplied and not voltage

  - The resistance in series with the source = 0

  - Thus v0=-isR

#### Non-Inverting Amplifier

- Designed to produce positive voltage gain

  - The voltage source is present in the positive inverting voltage terminal

  - The two equivalent resistors are in the same place as the ones in inverting op amps

V0=(1+RfR1)V0

- Special case: Voltage follower

  - The R1 branch is replaced with a short circuit

  - In this case the input = output V0= Vi

  - Two uses

    - Vi  can only source low current due to Vcc values whereas V0 can source much higher ones

    - Can be used as a buffer to isolate one stage of a circuit from another

#### Summing Op Amps

- Combines multiple inputs to make one common weighted output

- Uses superposition theory to determine the contribution of each source to the output

- Uses idea of superposition to make a common equation

  - v0=-RfR1v1+RfR2v2+RfR3v3

Example:

  - v0=-8201.5+8102+861.2

  - v0=-3.8V

#### Difference op amp

- Combination of an inverting and a non-inverting op amp

- Can be done using first principals

- Or use the equation:

  - v0=R2(1+R1R2)R1(1+R3R4)v2-R2R1v1

Example:

Design a difference op amp with inputs v1and v2 such that v0 = 3v2 – 5v1

R2R1=5

R1=1Ω    R2=5Ω

R21+R1R2R11+R3R4=3

5(1+15)1(1+R3R4)=3

R3R4=5(1+15)3-1

R3R4=1

R3=R4=5Ω

#### Cascading Op amps

- Is the head to tail arrangements of many op amps

- Each op has an amplification value – the ration of v0/vs

- Generally just the value of the of the resistor ratios

- For cascaded ones use the equation

# Chapter 6 – Conductors and Inductors

#### Capacitors

- Passive element designed to store energy in an electric field

- Consists of two conducting plates separated by an insulator

- Has a value of stored charge – capacitance

  - C=εAd

  - A - Surface area of the metal plate

  - ε – permittivity of the dielectric

  - d – distance between the plates

- Capacitance  - ratio of charge on one plate to the voltage difference between the two plates measured in Farads (F)

  - c=qv

- Thus the following equations are used to calculate current and voltage as neither are constant across the capacitor

  - it=cddtvt

    - I is positive if capacitor is charging and negative if discharging

  - vt=1Ct0titdt+vt0

- Energy and power

  - w=12cv2

  - p=vi=vCdvdt

- Capacitors are open circuits under DC conditions (capacitor is fully charged)

Example: The voltage source connected across a 10μF capacitor is given as vt=5sin200tV. Calculate i(t).

i=cdvdt

=10×10-6dvdt50sin2000t

=10×10-6×50 ×2000×cos2000t

=cos2000t

- Capacitors in series are added like resistors in parallel

  - 1Ceq=1c1+1c2+1c3

  - Thus voltage division in series works like current division in parallel

  - v1=c2c1+c2v

- Capacitors in parallel  - act like resistors in series

  - Ceq=c1+c2+c3

  - Thus current division works the same as voltage division in series

  - I1=c1c1+c2I

Example: Find the equivalent capacitance

=60s120+20s50+70

=40+20s120

=40μF

#### Inductors

- Passive element designed to store energy in its magnetic field

- Equation for physical characteristics – measured in Henries (H)

  - L=N2μAl

    - l – length of the coil

    - μ – permeability

    - A – cross sectional area

    - N – number of turns

- Voltage and current equations

  - vt=Lddtit

  - it=1Lt0tvtdt+it0

  - w=12Li2

  - p=vi=iLdidt

  - Is a short circuit under DC conditions

- Inductors in series are added like resistors in series

  - Ceq=c1+c2+c3

  - Thus voltage division in series works like voltage division in series

  - v1=c1c1+c2v

- Inductors in parallel  - act like resistors in parallel

  - 1Ceq=1c1+1c2+1c3

  - Thus current division works the same as current division in parallel

  - I1=c2c1+c2I

#### Calculating problems with conductors and inductors

- the equations for the inductors/ capacitors given

  - Do not remove anything in the circuit

  - Use simplification and or voltage/current division to isolate the desired element

  - Use the above equations to calculate the answers

- No equations are given

  - Use DC conditions and solve the circuit without the other elements

Example using DC conditions:

i=6mA

v=120V

Example using equations: If i1t=0.6e-2t and i(0) = 1.4A. Find i2(0), i2(t), i(t), v1(t), v2(t) and v(t)

vt=Lddtit

v1t=6ddt0.6e-2t

v1t=6-1.2e-2t

v1t=-7.2e-2t

it=1Lt0tvtdt+ it0

i2t=13t0t-7.2e-2tdt+it0
i2t=1.2e-2t-1+it0

i1+i2=i

i10+i20=i0

0.6e-20+i20=1.4

i20=0.8A

i2t=1.2e-2t-1+0.8A

i2t=1.2e-2t-0.4A

i1t=1.8e-2t+1.4A

v2t=8ddt1.8e-2t+1.4A

v2t=-28.8e-2tV

v1+v2=v

v=-36e-2tV

# Chapter 9 – Sinusoids and phasors

#### Complex Numbers

- Complex numbers have both a real and an imaginary part

- Appear in 3 forms

  - Rectangular - z=x+jy

    - X – real part

    - Y – imaginary part

  - Polar - z=r<φ

    - Similar to polar co-ordinates

    - R –combination of the real and imaginary magnitudes

    - Φ – the angle between r and the positive real axis

      - Must lie in (-180,180)

  - Exponential  - z=rejφ

    - Seldom used

- Conversions between types

  - Rectangular to polar

    - r=x2+y2

    - φ=tan-1yx

      - Counter clockwise is positive

  - Polar to exponential

    - Use the same variables

- Complex number arithmetic

  - Addition – use rectangular form

    - z=x1+jy1+x2+jy2

    - z=x1+x2+j(y1+y2)

  - Subtraction – same as addition

  - Multiplication – Use polar form

    - z=r1<φ×(r2<φ)

    - z=r1×r2<(φ+φ)

  - Division – same as multiplication

    - Subtract angles instead of add

  - Square root – use polar form

    - z=r1<φ2

  - Inverse – use polar form

    - 1z=1r1<-φ

  - Conjugate – use rectangular form

    - Change the sign before the imaginary part of the number

#### Sinusoids

- Signal in the form of a sine or cosine function

- v1(t)=vmsinωt-φ

  - ω – the angular frequency in radians/s

  - vm – the amplitude (maximum signal)

  - φ – the phase change of the function

- T=2πω

  - T – the period of the function (s)

- f=1T

  - F – the frequency of the function (Hz)

- To simplify some equations the compound angle trig equations my need to be used

  - sina±b=sinacosb±cosasinb

  - cosa±b=cosacosb∓sinasinb

- All sinusoids must be converted into the form: v1(t)=vmcosωt±φ

-sin(ωt+75) = cos(ωt+165)

-sin(ωt+75) = cos(ωt+165)

-sin(ωt)

-sin(ωt)

To do this use the following graph form

-sin(ωt-60) = cos(ωt+30)

-sin(ωt-60) = cos(ωt+30)

-cos(ωt)

-cos(ωt)

sin(ωt)

sin(ωt)

cos(ωt)

cos(ωt)

Example: Transform v=27sin3t-60 into standard form

v=-27cos3t+30

v=27cos3t-150

- Leading and lagging

  - Considering two vectors:

  - Use the smallest angle between the two

  - When moving clockwise from one vector to the other across the smallest vector:

    - The starting vector is lagging

    - The second is leading

- Converting sinusoids to into phasors

  - Change the phasor into standard form

  - v1(t)=vmcosωt±φ→vm<φ

  - dvdt=jωV

  - v dt=Vjω

#### Impedance and Admittance

- Admittance – the equivalent of resistance for AC circuits

  - Represented by Z and measured in Ohms

  - Same as resistance for resistors

  - All equations such as voltage division, wye to delta equations and series/parallel equations are exactly the same as the resistance ones

  - For inductors

    - Z=jωL

    - Using CIVIL – voltage leads current

  - For capacitors

    - z=1jwC

    - Using CIVIL – current leads voltage

- Impedance – equivalent of conductance for AC circuits

  - Represented by Y and measured in Siemens

  - Simply the inverse of admittance

- When working with circuits of multiple types of passive elements and AC current

  - Change all capacitance and inductance into admittance

  - Convert all sources into phasors

  - Look to find Zeq – likely needed for the problem

  - Treat the equation as one would a normal DC circuit

Example:

Example 2: Find Z