ORCAD/PSPICE Tutorial

Dr. George L. Engel
Department of Electrical and Computer Engineering
Southern Illinois University Edwardsville
Edwardsville, IL

September 3, 2003

Version 1.0

Students may obtain a student version of ORCAD and SPICE by visiting Dr. Engel's homepage at www.ee.siue.edu/~gengel. Click on theTeaching link and then onECE327.

Download the zip file and unzip it.

Install

the software by double clicking on the Setup icon.

When asked if you

wish to installSchematic orCapture, selectCapture.

After downloading and installing the demo version of ORRCAD, create a shortcut to the Capture program. Place the shortcut on your desktop to make it easy to launch the application. Also, create a folder somewhere on your drive where you can save your schematics and your PSPICE simulation files.

For example, here is the shortcut on my desktop. I called it ORCADdemo

ORCAD is a schematic editor.

In other words, it is a program that

allows you to draw circuit schematics.

The demo version that you have downloaded comes with a nice collection of parts libraries with parts that you are likely to use in student assignments and student laboratory exercises.

PSPICE is an electrical simulator that allows you to simulate the operation of the circuits whose schematic you prepared using ORCAD. This allows students to check for correct circuit operation even before they actually construct the circuit in lab.

The demo version of ORCAD

allows you to run PSPICE without even leaving the ORCAD user interface.

Launch the ORCAD schematic editor by double-clicking on theORCADdemo

icon.

Click onFile from the main menubar and then select Open New Project.

You should then see the following dialog box.

I decided to name my project "tutorial".

Notice I have entered the

Location of the folder where the schematics and my simulation results

will be saved.

Use theBrowse button to select the location whereyou

would like to saveyour files.

Make sure the Analog or Mixed A/D

option has been selected. You may then click the OK button.

The following dialog box will appear.

Make sure to select Create a blank project and then click the OK

button.

Let's create a simple voltage divider i.e. two resistors, each with a

value of 10KΩ.

We will drive the series combination with a sinusoid and observe the voltage at the node between the two resistors. Voltage division principles tell us that the voltage should be half of the input voltage.

From the menu at the top, selectPlace and thenPart (Shift-P if you

wish can be used as a shortcut). The following dialog box will appear.

Click on the Add Library button and choose the ANALOG library. You

will see the following.

Select the Ri.e. resistor component.

This can be done by double- clicking on the R. Place the component by clicking somewhere near the middle of the schematic page.

Click again and place R2.

You should

then hit theEsc key to exit the place mode.

The resistor R2 should

still be selected as is shown on the next page.

With resistor R2 selected, right mouse click on it.

A menu will

appear. Select theRotate option. You should observe the following.

Right mouse click on the 1k value on the R1 component.

A menu will

appear.

Select Edit Properties.

The dialog box shown below will

appear. Change the value to 10K.

Do the same for the R2 component.

At this point we should probably save our work. This can be done by
either typing Ctrl-S or by clicking on the diskette icon in the toolbar
or by going to theFile menu and selectingSave.

Do a Place Part again. This time add theSource library, From the Source library, selectVSIN. Place the sinewave generator and change the parameter values. VOFF should be 0. VAMPL should be set to 1. FREQ should be set to 1k.

Now place the ground symbol. ChoosePlace andGround. Let's use the
GND/CAPSYM symbol. Any of the grounds would work but this is my
personal favorite!

In order for PSPICE to simulate the circuit correctly, we must change

the name of the GND symbol from GND to 0.

This is because in all versions of SPICE, circuit ground must always be assigned a nodename of 0. We can do this by selecting the GND symbol. Now, right mouse click on the symbol. A menu will pop up. Select the Edit Properties option. In theName field where it says GND, change this to 0 as I have done below. Use a right mouse click on the Name field to highlight the field and click on display. The Display Properties dialog box will appear.

Your circuit should look like the figure below.

It is now time to wire the components to one another. This can be done

by placing a wire. Place Wire and Shift-w is the shortcut!

Click on a

pin then drag to mouse to another pin and click again.

When the components are all wired together, leave the wiring mode by

hitting theEsc key.

Your circuit should look like mine below.

We now are ready to simulate the circuit. We must first prepare a
simulation profile where we describe how we want the simulation to be
performed. To do this selectPSpice and then New Simulation Profile.

The following dialog box appears. I have entered a name for the
simulation. I decided to call it tran. You may call it anything you
wish. Click on theCreate button to create the profile.

After clicking on theCreate button, the following dialog box will
appear. Since the input frequency is 1 kHz which has a corresponding
period of 1 ms, I have decided to run for 5 msi.e. 5 periods of the
waveform. I also desire 100 data points per period of the sinusoid so
I have indicated that the maximum step size be 10µs (1µs / 100 = 10

µs)

We also need to tell PSPICE which nodes we would like to plot.

We do

this by going to the main menubar and selectingPSpice. We then select

Markersand Voltage.

We place the marker on the output of the voltage

divider as shown below.

For comparison purposes, let's also place a marker on the input node as

illustrated in the figure below.

At this point we should probably save our work. This can be done by
either typing Ctrl-S or by clicking on the diskette icon in the toolbar
or by going to theFile menu and selectingSave.

We are now ready to run the PSPICE simulation. We do this by selecting

PSpice from the main menubar and then Run. We could also click on the

the arrow icon on the toolbar. After a few seconds another window will

open and you will see the following simulation traces.

Notice the green trace (output) has one-half the amplitude of the red

trace (input) as we predicted.

Congratulations!!! You have successfully run PSpice to simulate a

simple voltage divider.