JPO 152 Assignment 4

Forces and Motion 2

Due date: 8th April 2013

| Surname: |  |
| --- | --- |
| Student No: |  |
| Field Of Study: |  |
| Group: |  |
| Total | /20 |

I am aware of the University of Pretoria’s plagiarism clause and steps that may be taken against me, should I be found guilty. This work is my original effort.

Signature: __________________

Question 1											[5]

A block is resting on an inclined plane with enough friction force in order to prevent the block from sliding down the plane. In order to start the block moving will it be easier to push the block down the slope or up the slope? Explain in detail your reasoning

If you attempt to push the block up the slope then friction will act down the slope hence just before motion starts according to newton’s first law the net force parallel to the plane will be zero. Hence:

Fapp-fs,max-Fgsinθ=0⇒Fapp=fs,max+Fgsinθ

However if you attempt to push the block down the slope then friction will act up the slope hence just as before:

Fapp+fs,max-Fgsinθ=0⇒Fapp=Fgsinθ-fs,max

Hence the applied force up the slope will be greater than the applied force down the slope hence it will be easier to cause motion down the slope

Question 2										            [15]

A 3 kg block rests on a floor. The coefficient of static friction between the block and the floor is 0.45 and the coefficient of kinetic friction between the block and the floor is 0.35

- What is the magnitude of the frictional force acting on the block from the floor? Explain											(2)

Zero. No attempted motion results in no friction force

- If a 11N force now acts on the block parallel to the floor, will the block move? Explain											(3)

No. fs,max=μsFN=0.45mg=13.23N hence the 11N force is smaller than the maximum static friction force and will not be enough to cause motion

- If the block was initially moving what magnitude force would need to be applied to the block in order to keep the block moving at constant velocity? Explain		(3)

For constant velocity newton’s first law applied hence

Fapp-fk=0⇒Fapp=μKFN=0.35mg=10.29N

- If situation (c) was maintained (i.e. the block was moving at constant velocity) and you increased the magnitude of the force you applied to a value greater than your answer in (c), then would the frictional force experienced by the block increase, decrease, or remain the same? Explain						(3)

Stay the same. fK is always a constant value regardless of the applied force.

- If a force acts on the block with a magnitude of 20N at an angle of 40° below the horizontal, will the block move? Explain						(4)

The force attempting to cause motion is Fapp,x=Fappcos40°=20cos40°=15N
In the y-direction there is no motion hence by Newton’s first law:

FN-Fg-Fappsin40°=0⇒FN=mg+20sin40°=42N

Hence fs,max=μsFN=0.4542=19N

Thus the force attempting to cause motion is less than fs,max and the box will not move